Land Surveying Final Project

SUR 2101C

Angel Alicea

July 17, 2014

Table of Contents

- Cover Sheet

  - Vicinity Map…………………………………………………….………………….……1

  - General Notes………………………………………………………………......………..2

  - Legend………………………………………………………………………...…………2

- Boundary Sheet

  - Field angles and distances………………………………………………………………3

  - Adjusted Angles……………………………………………………………….....……..4

  - Calculated Bearings…………………………………………………………....………..5

- Topographic Sheet

  - Location of all improvements on the site……………………………………………….6

- Bench Line Sheet

  - Adjusted Elevation………………………………………………………………………7

Vicinity Map:

N 76® 04’ 24” E

183.303’

N 89® 56’ 37” E

170.440’

N 06® 12’ 20” E

156.055’

N 05® 00’ 00” E

111.713’

General Field Notes:

- City of Survey - Tampa, Florida

- Location of Survey - University of South Florida Campus on the west side of the Engineering Kopp buildings property lines.

- Weather - Clear, Warm 80°F

- Date of Survey -June 12, 2014

- Time – 8:00 am

- Group – 8 people in the group

Legend:

Tree –

Light Pole -

Section 9, Township 28 South - Range 19 East Hillsborough County, Florida

Legal Description:

Section 9, Township 28 South - Range 19 East Hillsborough County, Florida. All lands in the state of Florida are measured from the Tallahassee meridian in longitude 84° 16’ 59”. Commencing at the portion of the SW ¼, Section 9, Township 28, South - Range 19 East, run East 183.303 feet, thence run South 156.055 feet to the point of beginning.

Thence, being said that along the N 05˚ 00’ 00’’ E bearing for 111.713 feet.  Thence, the bearings at N 76˚ 04’ 24’’ E for 183.3025 feet to the boundary located on the west side of the Engineering Kopp buildings property lines. Thence, at the bearings of S 06˚ 12’ 20’ W is approximately 156.055 feet along said boundary line to the North right of way line of USF Blvd. located after the boundary to the west of bearing 05˚ 00’ 00’’. Thence, for bearing N 89˚ 56’ 37’’ W is for 170.440 feet along the North right of way to the initial point of commencing.